# Acoustic Data Collection Protocol — Hanoi Urban Noise Study

Methodology replicates Nsumba et al. (2026, Scientific Data) — the Urban Noise Uganda 61K dataset — adapted to a two-collector, smartphone-based campaign in Hanoi.

# 1. General Strategy

Collection method: mobile (walking) smartphone-based sampling, following the protocol of the reference study — short georeferenced samples collected on foot, rather than fixed long-duration sensors. This maximizes spatial diversity, the critical input for the spatial noise model (each sample = one GPS point with its own urban-morphology context).

Tooling: ODK Collect (same data collection tool as the reference study) connected to a KoboToolbox server; one shared sound level meter app on both phones.

Per sample (~45–75 s): >= 10 s audio recording + sound pressure level (dB) + GPS (lat/lon/altitude/accuracy) + noise category + distance-to-road bracket + optional note.

Site mapping: walking circuits pre-selected with Google Maps/Earth and OSMnx, anchored on the typology points below.

Target volume: 20–30 samples per session; 300–700 samples total across all zones.

# 2. Zone Typologies & Walking Circuit Anchors

Each former "sensor location" becomes an anchor on a walking circuit. At each anchor, collect 4–6 samples at varying distances from the source (0–10 m, 10–30 m, 30–60 m, > 60 m / behind buildings) before moving to the next anchor. Distance variation is deliberate: the model learns how noise decays with distance and shielding.

## Zone 1 — Ocean Park (New Urban Fabric)

Focus: sound propagation, high-rise shielding effect, construction noise.

| Anchor | Description |
| --- | --- |
| 1. Urban Canyon | Main avenue in front of the Masterise towers — continuous traffic in canyon geometry |
| 2. Shield Zone | Adjacent street/courtyard directly behind the buildings — acoustic protection |
| 3. Intersection | Major traffic light ~300–400 m from Anchor 1 — congestion and acceleration noise |
| 4. Construction | Near an active construction site — acoustic footprint of urban development |

## Zone 2 — Vinh Tuy (Transport Infrastructure)

Focus: heavy, continuous, high-speed flows (free-flow conditions).

| Anchor | Description |
| --- | --- |
| 1. Source Zone | Long straight section of Vanh Dai 2, away from lights/intersections — maximum emission |
| 2. Transition Zone | Near the T5 entry ramp — vehicle diffraction and insertion noise |
| 3. Continuous Corridor | Long straight section — stable uninterrupted traffic |

## Zone 3 — Hoan Kiem (Old Quarter)

Focus: narrow streets, mixed use, pedestrian zones.

| Anchor | Description |
| --- | --- |
| 1. Pedestrian-Traffic Interface | Boundary where vehicle traffic meets high pedestrian activity |
| 2. Dense Corridor | Narrow, highly built-up streets — reverberation, confined geometry |
| 3. Lake Corridor | Along the lake — transition from traffic noise to open-space acoustics |

# 3. Temporal Planning

Daily capture window: 05:00 to 23:00 — any session within this window is valid and contributes to the hourly-cycle analysis.

Priority time slots (4 sessions/day when possible): 08:00, 12:00, 17:00, ~22:00. Rush-hour windows (08:00–10:00, 16:00–18:00) are priority slots. Late-evening sessions (after ~21:00) in pairs only (the reference study’s main limitation was night-time coverage due to safety; we mitigate but document the same constraint).

Weekday vs weekend: each zone covered on at least 1 weekday and 1 weekend day.

Rotation: with 2 collectors, cover 2 zones in parallel per session day; rotate so each zone accumulates all 4 time slots within 2–3 weeks.

Weather: no collection during rain (alters both traffic and propagation; also the reference study’s limitation #2). Rainy-day sessions are rescheduled.

# 4. Hardware & Measurement Protocol

Devices: 2 Android smartphones, ODK Collect + one shared sound meter app.

Cross-calibration: before the campaign, both phones side by side against the same source at 3 levels; systematic offsets recorded and corrected in analysis. Reference: the original study validated smartphone SPL against a Class 1 Casella CEL-633A1 meter with < 2 dB error over 35–120 dB (their Table 1); we cite this and note the absence of a Class 1 reference as a limitation.

Handling: phone held at arm’s length, ~1.2 m above ground, mic unobstructed, collector silent during recording.

Sequence per sample:

1. Open ODK form

2. Wait for GPS accuracy < 10 m

3. Record >= 10 s audio

4. Read dB from sound meter app immediately after (quasi-stationary ambient assumption over the ~30 s window; transient events noted in the form)

5. Enter dB + category + distance bracket

6. Submit, then walk 30–60 s to the next spot

# 5. Metadata

| Metadata | How collected |
| --- | --- |
| Date & time | Automatic (ODK timestamp) |
| GPS + accuracy | Automatic (ODK geopoint) |
| Weather (temp, wind, precipitation, humidity) | Open-Meteo API from timestamps, merged during analysis — no field entry needed |
| Spatial context (building density, road density, intersections, distance to road) | Computed post-hoc from OpenStreetMap via OSMnx, per the reference study’s morphology metrics (R = 300 m) |
| Notable events | Free-text note field in the ODK form |

# 6. Data Flow

ODK Collect (phone) → KoboToolbox server → CSV export → cleaning + weather enrichment + standards analysis (QCVN 26:2010/BTNMT, WHO) → transfer learning (Uganda-pretrained model + our measurements) → predicted noise map → GAMA scenario simulation.